STM32H753ZI – Memory Map:

Memory Layout:

| Region | Start | Size | Usage |
| --- | --- | --- | --- |
| Flash | 0x08000000 | 2MB | Code, Constants |
| ITCM-RAM | 0x00000000 | 64KB | Time critical code |
| DTCM-RAM | 0x20000000 | 128KB | Stack, Heap, Data |
| AXI SRAM | 0x24000000 | 512KB | General Data |
| Backup-SRAM | 0x38000000 | 4KB | Always-on data |

Linker Sections:

| Section | Description |
| --- | --- |
| .isr_vector | Interrupt table |
| .text | Code |
| .rodata | Constants |
| .data | Initialized variables |
| .bss | Zero-initialized |

Stack Placement:

_estack = ORIGIN(RAM) + LENGTH(RAM)